# Dynamic molecular crystals for organic optoelectronics

## Kalyan Jyoti Kalita1,2,3

### 1Indian Institute of Science Education and Research (IISER) Kolkata, Mohanpur, Nadia - 741246, West Bengal, India

### 2Indian Institute of Technology (IIT) Hyderabad, Kandi, Sangareddy - 502284, Telangana, India

### 3Present address: University of Lincoln, Brayford Pool, Lincoln LN6 7TS, United Kingdom

### kkalita@lincoln.ac.uk

Dynamic molecular crystals represent a compelling class of smart materials poised to revolutionize organic optoelectronics. These crystalline organic solids challenge the traditional notion of crystals as rigid and static by demonstrating capabilities such as bending, twisting, shape memory, and self-healing.1-3 Integrating dynamic materials with rich optical and charge transport functionalities will be ideal candidates for flexible, responsive, durable, and sustainable optoelectronic device architectures, viz., organic field effect transistors (OFETs), organic light emitting diodes (OLEDs), and smart sensors. By leveraging non-covalent interactions (e.g., π–π stacking, van der Waals forces, hydrogen bonding) and quantum mechanical principles, we engineer crystal architectures capable of harvesting triplet excitons through phosphorescence and thermally activated delayed fluorescence (TADF) (Figure 1).4-6 Our work explores the rational design of organic molecular crystals that exhibit dynamic mechanical responses while maintaining high photoluminescence quantum yield (PLQY) and tuneable emission characteristics. Moreover, dynamic materials with high charge carrier mobility7 are particularly promising for applications in OFETs, photodetectors, and flexible electronics. We also examine polymorphic and co-crystalline systems where dynamic phase transitions induce mechanical reconfiguration.8-11 Highlighting recent advances in crystal adaptronics,1-3 we demonstrate how these soft, yet robust, molecular solids enable next-generation applications, including optical waveguides and stimuli-responsive actuators. This poster will emphasise the synergy between crystal engineering and photophysical/charge transport behaviours, offering a strategy for integrating organic crystalline materials into real-world optoelectronic devices.

###### Figure 1. Schematic summary of the findings: a) graphical representation of the dynamic phosphorescent material, b) simplified diagram representing our non-covalent donor-acceptor design strategy for dynamic TADF material.

#### [1] Naumov, P., Karothu, D. P., Ahmed, E., Catalano, L., Commins, P., Mahmoud Halabi, J., Al-Handawi, M. B. & Li, L. (2020). J. Am. Chem. Soc. 142, 13256.

#### [2] Commins, P., Al-Handawi, M. B. & Naumov, P. (2025). Nat. Rev. Chem. 9, 343.

#### [3].Bhunia, S., Chandel, S., Karan, S. K., Dey, S., Tiwari, A., Das, S., Kumar, N., Chowdhury, R., Mondal, S., Ghosh, I., Mondal, A., Khatua, B. B., Ghosh, N. & Reddy, C. M. (2021). Science 373, 321.

#### [4] Kalita, K. J., Mondal, S., Reddy, C. M. & Vijayaraghavan, R. K. (2023). Chem. Mater. 35, 709.

#### [5] Kalita, K. J., Mondal, S., Reddy, C. M. & Vijayaraghavan, R. K. (2023). Chem. Sci. 14, 13870.

#### [6] Kalita, K. J., Arikkottira M, R. & Vijayaraghavan, R. K. (2024). CrystEngComm 26, 4751.

#### [7] Kalita, K. J., Giri, I. & Vijayaraghavan, R. K. (2021). RSC Adv. 11, 33703.

#### [8] Kalita, N., Kalita, K. J., Borah, S. S., Puschmann, H., Nath, N. K. & Thakuria, R. (2025). ACS Appl. Opt. Mater. 3, 1078.

#### [9] Kalita, N., Deka, P., Ghosh, I., Kalita, K. J., Gogoi, A., Malla Reddy, C. & Thakuria, R. (2025). J. Mater. Chem. C 13, 10769.

#### [10] Deka, P., Das, S., Sarma, P., Kalita, K. J., Vijayaraghavan, R. K., Reddy, C. M. & Thakuria, R. (2024). Crys. Growth Des. 24, 2322.

#### [11] Ghosh, T., Birudula, S., Kalita, K. J. & Vijayaraghavan, R. K. (2020). Chem. Eur. J. 26, 10501.

The author of this poster acknowledges his mentors, Dr. Ratheesh K Vijayaraghavan, Prof. C Malla Reddy, Dr. Annadhasan M, Prof. Len J. Barbour, Dr. Ranjit Thakuria for their support.